FP0 內部特定用途地址名稱及用法

R9013	起動 ON 脈沖			只在第一個程式遁環中閉合，跟着轉為開路

Turns on only at the first scan in the operation.

Turns off from the second scan and maintains the off state.

R9014  	起動 OFF 脈沖			只在第一個程式遁環中開路，跟着轉為閉合

Turns off only at the first scan in the operation.

Turns on from the second scan and maintains the on state.

R9015	Step ladder initial on pulse relay 	Turns on for an instant only in the first scan of the

process the moment step ladder process is opened.

R9016,

R9017					(沒有用) Not used

R9018	0.01 秒 時鐘脈波			以 0.01 秒遁環 on/off 動作

0.01 s

R9019 	0.02  秒 時鐘脈波			以 0.02 秒遁環 on/off 動作.

0.02 s

R901A	0.1 秒 時鐘脈波			以 0.1 秒遁環 on/off 動作.

0.1s

R901B 	0.2 秒 時鐘脈波			以 0.2 秒遁環 on/off 動作

0.2 s

R901C	1 秒 時鐘脈波			以 1 秒遁環 on/off 動作.

1 s

R901D	2 秒 時鐘脈波			以 2 秒遁環 on/off 動作.

2 s

R901E    1 min  時鐘脈波			以 1 分鐘 遁環 on/off 動作.

1 min.

R901F					 Not used

R9020	RUN mode flag			Turns off while the mode selector is set to PROG.

Turns on while the mode selector is set to RUN.

R9021 to

R9025					Not used

R9026 	Message flag 			Turns on while the F149 (MSG) instruction is executed.

R9027	Remote mode flag			Turns on while the mode selector is set to REMOTE.

R9028					Not used

R9029	Forcing flag 			Turns on during forced on/off operation for I/O relay and timer/counter contacts.

R902A	External interrupt enable flag	Turns on while the external interrupt trigger is enabled by the ICTL instruction.

R902B     Interrupt error flag			Turns on when an interrupt error occurs.

R902C to

R9031					Not used

R9032 	RS232C port mode flag		When “General-use port” is selected, ”K2” goes on.

R9033 	Printout instruction flag		Turns on while a F147 (PR) instruction is executed.

Turns off when a F147 (PR) instruction is not executed.

R9034 Rewrite during RUN flag  		This is a special internal relay that goes on for only the first scan following

the completion of rewriting in the RUN mode. (CPU Ver. 2.1 or later available)

R9035 S-LINK I/O communication error flag	Turns on when the S-LINK error (ERR 1, 3 or 4) occurs using S-LINK system.

R9036 S-LINK communication status flag	Turns on when communication is taking place with an S-LINK input/output

unit.

R9037 RS232C communication error flag	Turns on when the serial data communication error occurs.

R9038 RS232C reception completed flag	Turns on when a terminator is received during the serial data communicating.

R9039 RS232C transmission completed flag	Turns on while data is not send during the serial data communicating.

Turns off while data is being sent during the serial data communicating.

R903A High-speed counter control flag for ch0	Turns on while the high-speed counter instruction “F166 (HC1S) to F170

(PWM)” is executed.

R903B High-speed counter control flag for ch1	Turns on while the high-speed counter instruction “F166 (HC1S) to F170

(PWM)” is executed.

R903C High-speed counter	control flag for ch2 Turns on while the high-speed counter instruction “F166 (HC1S) to F170

(PWM)” is executed.

R903D High-speed counter control flag for ch3	 Turns on while the high-speed counter instruction “F166 (HC1S) to F170

(PWM)” is executed.

R903E,

R903F					Not used