NAiS PLC FP0 / FP-X

指令結構

基本指令 Basic Instructions

ST         START		Begins a logic operation with a Form A (normally open) contact.

ST/       START NOT	Begins a logic operation with a Form B (normally closed) contact.

OT        OUT 		Outputs the operated result to the specified output.

/            NOT		Inverts the operated result up to this instruction.

AN        AND		Connects a Form A (normally open) contact serially.

AN/	AND NOT	Connects a Form B (normally closed) contact serially.

OR       OR		Connects a Form A (normally open) contact in parallel.

OR/      OR NOT	Contacts a Form B (normally closed) contact in parallel.

ANS     AND STACK	Connects the multiple instruction blocks serially.

ORS	OR STACK	Connects the multiple instruction blocks in parallel.

PSHS	Push stack        Stores the operated result up to this instruction.

RDS	Read stack        Reads the operated result stored by the PSHS instruction.

POPS   Pop stack           Reads and clears the operated result stored by the PSHS instruction.

DF       Leading edge differential    Turns on the contact for only one scan when the leading edge of the trigger is detected.

DF/      Trailing edge differential     Turns on the contact for only one scan when the trailing edge of the trigger is detected.

SET	SET			Output is set to and held at on.

RST	RST			Output is set to and held at off.

KP	KEEP			Outputs at set trigger and holds until reset trigger turns on.

NOP     No operation.

TML	0.001s on-delay Timer     Afetr set value [n](*) x 0.001 seconds, timer contact [a] is set to on.

TMR	0.01s on-delay Timer       Afetr set value [n](*) x 0.01 seconds, timer contact [a] is set to on.

TMX	0.1s on-delay Timer	  Afetr set value [n](*) x 0.1 second, timer contact [a] is set to on.

TMY	1s on-delay Timer             Afetr set value [n](*) x 1 second, timer contact [a] is set to on.

CT	COUNTER                        Decrements from the preset value [n](*).

SR	Shift register		Shifts one bit of 16-bit [word internal relay(WR)] data to the left.

MC	Master control relay end

MCE	Master control relay

JP	Jump-Label       The program jumps to the label instruction and continues from there.

LBL      Label

LOOP	Loop-Label

The program jumps to the label instruction and continues from there (the number of jumps is set in [S]).

BRK	BREAK

Stops program execution when the predetermined trigger turns on in the TEST/RUN mode only.

ED        END

CNDE	Conditional end

The operation of program is ended when the trigger turns on.

SSTP	Start step

The start of program [n] for process control.

NSTP	Next step

Starts the specified process [n] and clears the process currently started. (Pulse execution type)

NSTL	Next step

Starts the specified process [n] and clears the process currently started. (Scan execution type)

CSTP	Clear step

STPE	Step end	End of step ladder area

CALL

SUB

RET

INT

IRET

ICTL

ST =		ST <>		ST >		ST >=		ST <		ST <=

STD = 		STD <>		STD >		STD >=		STD <		STD <=

AN =		AN <>		AN >		AN >=		AN <		AN <=

AND = 		AND <>		AND >		AND >=		AND <		AND <=

OR =		OR <>		OR >		OR >=		OR <		OR <=

ORD =		ORD <>	ORD >		ORD >=	ORD <		ORD <=

數據傳送指令  Data Transfer Instruction

F0 (MV) 	16-bit data move     	Transfers 16-bit data of specified area No.    (S)=>(D)

F1 (DMV) 	32-bit data move

F2 (MV/) 	16-bit data invert and move

F3 (DMV/)	32-bit data invert and move

F5 (BTM)	Bit data move

The specified one bit in [S] is transferred to the specified one bit in [D]. The bit is specified by [n].

S:	WX, WY, WR, WL, SV, EV, DT, LD, FL, IX, IY, constant K and constant H

Index modification is possible.

n:	WY, WR, WL, SV, EV, DT, LD, FL, IX, IY, constant K and constant H

Index modification is possible.

D:	WY, WR, WL, SV, EV, DT, LD, FL, IX and IY

Index modification is possible.

F6 (DGT) 	Hexadecimal digit (4-bit) data move

The specified one digit in [S] is transferred to the specified one digit in [D]. The digit is specified by [n].

F10 (BKMV) 	Block move

The data between [S1] and [S2] is transferred to the area starting at [D].

F11 (COPY) 	Block copy

The data of [S] is transferred to the all area between [D1] and [D2].

F15 (XCH) 	16-bit data exchange

Exchanges 16-bit data in 2 areas.

(D1)=>(D2), (D2)=>(D1)

F16 (DXCH) 	32-bit data exchange

Exchanges 32-bit data in 2 areas.

(D1+1, D1)=>(D2+1, D2)

(D2+1, D2)=>(D1+1, D1)

F17 (SWAP)	Higher/lower byte in 16-bit data exchange

Exchanges the higher byte and lower byte of [D].

F19	Auxiliary jump-Label      The program jumps to the label instruction specified by [S] and continues from there.

二進數學運算指令  Binary Arithmetic Instructions (F20 –F38, F160)

F20 (+)		16-bit data addition

Adds 16-bit data.

(D)+(S)=>(D)

F21 (D+)  	32-bit data addition

F22 (+)		16-bit data addition

F23 (D+) 	32-bit data addition

F25 ( - ) 	16-bit data subtraction

F26 (D- )	32-bit data subtraction

F27 ( - )		16-bit data subtraction

Subtracts 16-bit data.

(S1)-(S2)=>(D)

F28 (D- )	32-bit data subtraction

Subtracts 32-bit data.

(S1+1, S1)-(S2+1, S2)=>(D+1, D)

F30 (*)		16-bit data multiplication

Multiplies 16-bit data.

(S1) x (S2)=>(D+1, D)

F31(D*) 	32-bit data multiplication

Multiplies 32-bit data.

(S1+1, S1) x (S2+1, S2)=>(D+3, D+2, D+1, D)

F32 (%)		16-bit data division

Devides 16-bit data.

(S1)/ (S2) =>	quotient(D)

remainder(DT9015)

*	With the FP10/FP10S/FP10SH/FP2, remainder is stored in DT90015.

F33 (D%) 	32-bit data division

Devides 32-bit data.

(S1+1, S1)/(S2+1, S2) =>	quotient(D+1, D)

remainder(DT9016, DT9015)

*With the FP10/FP10S/FP10SH/FP2, remainder is stored in DT90016 and DT90015.

F35 (+1) 	16-bit data increment

Adds 1 to 16-bit data.

(D)+1=>(D)

F36 (D+1) 	32-bit data increment

Adds 1 to 32-bit data.

(D+1, D)+1=>(D+1, D)

F37 ( -1) 	16-bit data decrement

Subtracts 1 from 16-bit data.

(D)-1=>(D)

F38 (D-1) 	32-bit data decrement

Subtracts 1 from 32-bit data.

(D+1, D)-1=>(D+1, D)

BCD碼運算指令  BCD Arithmetric Instructions (F40 – F58)

F40 (B+) 	4-digit BCD data addition

Adds 4-digit BCD data.

(D)+(S)=>(D)

F41 (DB+) 	8-digit BCD data additionF42 (B+)

Adds 8-digit BCD data.

(D+1, D)+(S+1, S)=>(D+1, D)

F43 (DB+) 	8-digit BCD data addition

F45 (B- ) 	4-digit BCD data subtraction

F46 (DB- ) 	8-digit BCD data subtraction

Subtracts 8-digit BCD data.

(D+1, D)-(S+1, S)=>(D+1, D)

F47 (B- ) 	4-digit BCD data subtraction

Subtracts 4-digit BCD data.

(S1)-(S2)=>(D)

F48 (DB- ) 	8-digit BCD data subtraction

Subtracts 8-digit BCD data.

(S1+1, S1)-(S2+1, S2)=>(D+1, D)

F50 (B*) 	4-digit BCD data multiplication

Multiplies 4-digit BCD data.

(S1) x (S2)=>(D+1, D)

F51 (DB*) 	8-digit BCD data multiplication

Multiplies 8-digit BCD data.

(S1+1, S1) x (S2+1, S2)=>(D+3, D+2, D+1, D)

F52 (B%)	4-digit BCD data division           Divides 4-digit BCD data.    (S1)/(S2) =>	quotient(D)

remainder(DT9015)

- With the FP10/FP10S/FP10SH/FP2, remainder is stored in DT90015.

F53 (DB%)	8-digit BCD data division	Divides 8-digit BCD data.	(S1+1, S1)/(S2+1, S2) =>	quotient(D+1, D)

remainder(DT9015, DT9016)

- With the FP10/FP10S/FP10SH/FP2, remainder is stored in DT90016 andDT90015.

F55 (B+1) 	4-digit BCD data increment	Adds 1 to 4-digit BCD data.   (D)+1=>(D)

F56 (DB+1) 	8-digit BCD data increment	Adds 1 to 8-digit BCD data.   (D+1, D)+1=>(D+1, D)

F57 (B-1) 	4-digit BCD data decrement	Subtracts 1 from 4-digit BCD data.	(D)-1=>(D)

F58 (DB-1)	8-digit BCD data decrement	Subtracts 1 from 8-digit BCD data.  (D+1, D)-1=>(D+1, D)

數據比較指令 Data Compare Instruction

F60 (CMP)	16-bit data compare

Compares two specified 16-bit data and outputs the result to special internal relay.						(S1)>(S2)  =>R900A : ON

(S1)=(S2)  =>R900B : ON

(S1)<(S2)  =>R900C : ON

S1:	WX, WY, WR, WL, SV, EV, DT, LD, FL, IX, constant K and constant H

Index modification is possible.

S2:	WX, WY, WR, WL, SV, EV, DT, LD, FL, IX, constant K and constant H

Index modification is possible.

F61 (DCMP)	32-bit data compare

Compares two specified 32-bit data and outputs the result to special internal relay.

(S1+1, S1)>(S2+1, S2)  =>R900A : ON

(S1+1, S1)=(S2+1, S2)  =>R900B : ON

(S1+1, S1)<(S2+1, S2)  =>R900C : ON

F62 (WIN) 	16-bit data band compare

Compares two 16-bit data bands with codes and outputs the result to special internal relay.

(S1)>(S3)        	=>R900A : ON

(S2)<=(S1<=(S3)	=>R900B : ON

(S1)<(S2)        	=>R900C : ON

F63 (DWIN)	32-bit data band compare

F64 (BCMP)	Block data compare

Compares the two blocks beginning with [S2] and [S3] to see if they are equal.

邏輯運算指令 Logic Operation Insturctions

F65 (WAN)	16-bit data AND		Performs bit-wise AND operation on two 16-bit data items.	(S1) AND (S2) => (D)

F66 (WOR)	16-bit data OR		Performs bit-wise OR operation on two 16-bit data items.	(S1) OR (S2) => (D)

F67 (XOR) 	16-bit data exclusive OR   Performs bit-wise exclusive OR operation on two 16-bit data items.

{(S1) AND NOT(S2)} OR {NOT(S1) AND (S2)} => (D)

F68 (XNR) 	16-bit data exclusive NOR

數據傳換指令 Data Conversion Structions (F70 – F96)

F70 (BCC) 	Block check code calculation

F71 (HEXA) 	Hexadecimal data => ASCII code

F72 (AHEX) 	ASCII code  => Hexadecimal data

F73 (BCDA) 	4-digit BCD data => ASCII code

F74 (ABCD)	ASCII code  => 4-digit BCD data

F75 (BINA) 	16-bit binary data => ASCII code

F76 (ABIN) 	ASCII code  =>16-bit binary data

F77 (DBIA) 	32-bit binary data  => ASCII code

F78 (ADBI) 	ASCII code  =>32-bit binary data

Converts the ASCII code specified by [S1] and [S2] to 32 bits of binary data and stores it in [D+1, D].

F80 (BCD) 	16-bit binary data => 4-digit BCD data

Converts the 16 bits of binary data specified by [S] to four digits of BCD data and stores it in [D].

Example) K100 => H100

F81 (BIN) 	4-digit BCD data  => 16-bit binary data

Converts the four digits of BCD data specified by [S] to 16 bits of binary data and stores it in [D].

Example) H100 => K100

F82 (DBCD) 	32-bit binary data  =>8-digit BCD data

Converts the 32 bits of binary data specified by [S+1, S] to eight digits of BCD data and stores it in [D+1, D].

F83 (DBIN) 	8-digit BCD data  =>32-bit binary data

F84 (INV) 	16-bit data invert (complement of 1)	Inverts each bit of data of [D].

F85 (NEG) 	16-bit data invert (complement of 2)	Inverts each bit of data of [D] and adds 1 (inverts the sign).

F86 (DNEG) 	32-bit data invert (complement of 2)

F87 (ABS)	16-bit data absolute	Gives the absolute value of the data of [D].

F88 (DABS) 	32-bit data absolute	Gives the absolute value of the data of [D+1, D].

F89 (EXT) 	16-bit data sign extension	Extends the 16 bits of data in D. to 32 bits in [D+1, D].

F90 (DECO) 	Decode		Decodes part of the data of [S] and stores it in [D]. The part is specified by [n].

F91 (SEGT) 	7-segment decode	Converts the data of [S] for use in a 7-segment display and stores it in (D+1, D).

F92 (ENCO) 	Encode			Encodes part of the data of [S] and stores it in [D]. The part is specified by [n].

F93 (UNIT) 	16-bit data combine

The least significant digit of each of the [n] words of data beginning at [S] are stored (united) in order in [D].

F94 (DIST) 	16-bit data distribute

Each of the digits of the data of [S] are stored in (distributed to) the least significant digits of the areas beginning at [D].

F95 (ASC) 	Character => ASCII code

Twelve characters of the character constants of [S] are converted to ASCII code and stored in [D] to [D+5].

F96 (SRC) 	16-bit table data search

The data of [S1] is searched for in the areas in the range [S2] to [S3] and the result is stored in DT9037 and DT9038.

*	With the FP10/FP10S/FP10SH/FP2, the result is stored in DT90037 and DT90038.

數據移位指令 Data shift Instructions (F98 – F113)

F98 (CMPR)	Data table shift-out and compress

Transfers [D2] to [D3]. Any parts of the data between [D1] and [D2] that are 0 are compressed, and shifted in order toward [D2].

F99 (CMPW)	Data table shift-in and compress

Transfers [S] to [D1]. Any parts of the data between [D1] and [D2] that are 0 are compressed, and shifted in order toward [D2].

F100 (SHR) 	Right shift of multiple bits (n bits) in a 16-bit data		Shifts the [n] bits of [D] to the right.

F101 (SHL) 	Left shift of multiple bits (n bits) in a 16-bit data		Shifts the [n] bits of [D] to the left.

F105 (BSR)	Right shift of one hexadecimal digit (4-bit)

Shifts one hexadecimal digit (4 bits) of the specified 16-bit data to the right.

F106 (BSL) 	Left shift of one hexadecimal digit (4-bit)

Shifts one hexadecimal digit (4 bits) of the specified 16-bit data to the left.

F110 (WSHR)	Right shift of one word (16-bit)

Shifts the one word of the areas by [D1] and [D2] to the right.

F111 (WSHL)	Left shift of one word (16-bit)		Shifts the one word of the areas by [D1] and [D2] to the left.

F112 (WBSR) 	Right shift of one hexadecimal digit (4-bit)

Shifts the one digit of the areas by [D1] and [D2] to the right.

F113 (WBSL) 	Left shift of one hexadecimal digit (4-bit)

Shifts the one digit of the areas by [D1] and [D2] to the left.

基本功能指令  Basic Function Instructions (F118, 119, 137 )

F118 (UDC)	UP/DOWN counter   Increments or decrements from the preset value [S] based on up/down input.

F119 (LRSR)	Left/right shift register

Shifts one bit of 16-bit data range specified by [D1] and [D2] to the left or to the right.

F137 (STMR)	Auxiliary timer (16-bit)

數據遁環移位指令  Data Rotate Instructions (F120 – F123)

F120 (ROR)	Rotates the [n] bits in data of [D] to the right.

F121 (ROL) 	Rotates the [n] bits in data of [D] to the left.

F122 (RCR) 	Rotates the [n] bits in 17-bit area consisting of [D] plus the carry flag (R9009) data to the right.

F123 (RCL) 	Rotates the [n] bits in 17-bit area consisting of [D] plus the carry flag (R9009) data to the left.

位元操作指令  Bit Manipulation Intructions (F130 – F136 )

F130 (BTS) 	16-bit data bit set

F131 (BTR)	16-bit data bit reset

F132 (BTI) 	16-bit data bit invert

F133 (BTT) 	16-bit data bit test

F135 (BCU) 	Number of ON (1) bits in 16-bit data	Stores the number of ON bits in the data of [S] in [D].

F136 (DBCU) 	Number of ON (1) bits in 32-bit data	Stores the number of ON bits in the data of [S+1, S] in [D].

F137 (STMR)   Auxiliary timer     After set value [S] x 0.01 seconds, the specified output(*) and R900D are set to on.

F183 (DSTM)

F140 (STC) 	Carry flag (R9009) set

F141 (CLC) 	Carry flag (R9009) reset

F143 (IORF) 	Partial I/O update

F147 (PR) 	Printout

F148 (ERR) 	Self-diagnostic error set

Stores the self-diagnostic error number [n] in DT9000 or DT90000, turns R9000 on, and turns on the ERROR LED.

*	With the FP10/FP10S/FP10SH/FP2, the self-diagnostic error number [n] is stored in DT90000.

F149 (MSG)	Message display

Displays the character constant of [S] in the connected programming tool.

特殊指令 Special Instructions (high speed counter) F162 – F176

F166 (HC1S) 	ON when the target value agrees (with channel specification)

Turns the output Yn ON when elapsed values of the internal high speed counter reaches the target value of [S+1,S].

F167 (HC1R)	OFF when the target value agrees (with channel specification)

Turns the output Yn OFF when elapsed values of the internal high speed counter reaches the target value of [S+1,S].

F168 (SPD1) 	Motion control(with channel specification)

Outputs the motion pulse through the specified port (Y0, Y1) corresponding to the contents of the data table starting with [S].

F169 (PLS) 	Pulse output instruction (with channel specification)

Outputs the pulse string through the specified port (Y0, Y1) corresponding to the contents of the data table starting with [S].

F170 (PWM) 	PWM output instruction (with channel specification)

Outputs PWM through the specified port (Y0, Y1) corresponding to the contents of the data table starting with [S].