PLCs 指令對照  Instruction  Mapping ( 基本指令Basic Instructions )

This List provides a guide line for converting instructions between PLCs, Please refer to the corresponding PLC programming manual for details.

|  | Omron (SP20, C20) | NAiS ( FP0, FP2 ) | Mitsubishi ( 三菱FX-2N ) | ### Simens ### (西門子S7-200) | ### Alen-Bradley (SLC-500) | ### Schneider ### TSX Nano |
| --- | --- | --- | --- | --- | --- | --- |
| Load | LD | ST | LD | LD | XIC (Examine if Close) | LD |
| Load Not | LD NOT | ST/ | LDI | LDN | XIO (Examine if Open) | LDN |
| And | AND | AN | AND | A |  | AND |
| And Not | AND NOT | AN/ | ANI | O |  | ANDN |
| And Load | AND LD | ANS | ANB | AN |  | ( ) |
| Or | OR | OR | OR | ON |  | OR |
| Or Not | OR NOT | OR/ | ORI | ALD |  | ORN |
| Or Load | OR LD | ORS | ORB | OLD |  | ( ) |
| Out | OUT | OT | OUT | = | OTE | ST |
| Out Not | OUT NOT | OT/ | - |  |  | STN |
| Branch | IL | PSHS RDS POPS | MPS MRD MPP | LPS (Logic Push) LRD (Logic Read) LPP (Logic Pop) AENO (And ENO) LDS (Load Stack) |  | MPS MRD MPP |
| TIMER | TMR n, time | TML n, Ktime TMR n, Ktime TMX n, Ktime TMY n, Ktime | OUT Y,M,S OUT Tn Ktime OUT Cn Ktime (T0-99 100ms) | TON Tn Ktime (On Delay Timer) TOF Tn Ktime (Off Delay Timer) TONR Tn Ktime (On time remember Delay Timer) | TON Tn:n Preset9999 Accum9999 | TM |
| COUNTER Updown counter | CNT n, count | CT n, Kcount UDC | OUT Cn Kcount | CTU (count up) CTD (count down) CTUD (up/down) | CTU(count up) CTD(count down) | CU(count up) CD(count down) |
| Keep | KEEP | KP | - | SR RS |  |  |
| Set Reset |  | SET RST | SET Y,M:1 S, special M coils:2 RST Y,M,S,D,V,Z RST T,C | S R | OTL (Turn a bit ON) OTU (Turn a bit Off) | S R |
| Leading Edge differential Trailing Edge differential |  | DF DF/ | LDP ANP ORP LDF ANF ORF | P N | OSR | LDR ANDR ORR LDF ANDF ORF |
| Leading Edge differential Trailing Edge differential | DIFU DIFD | DFI | PLS Y,M PLF Y,M |  |  |  |
| Master Control | STEP | MC MCE | MC MCR |  |  | MCR MCS |
| No operation | NOP | NOP |  |  |  |  |
| End | END | ED | END |  |  | END |