PLC Basic Instruction Compare

### Omron			Mitsubishi		Matsushita		Simens			Alen-Bradley

### (SP20,C20)		(FX0, FX2)		(FP0, FP1, FP3)	Simatic S7		SLC-500

# LD			LD			ST			LD		    XIC (Examine if Close)

LD  NOT		LDI			ST/			LDN		    XIO (Examine if Open)

# AND			AND			AN			A		    OTE (Output Engergize)

OR			OR			OR			O

AND NOT		ANI			AN/			AN

OR  NOT		ORI			OR/			ON		    OSR (Trigger a one time event)

AND LD		ANB			ANS			ALD

OR  LD		ORB			ORS			OLD

OUT			OUT			OT			=

OUT NOT		-			-

TIM address  #time	OUT Tn  Ktime		TMX, TMY		TON  Tn K100   TON Tn:n Preset9999 Accum9999

CNT address #count	OUT Cn  Kcount		CT					    CTU (count up)

CTD (count down)

Keep			Set								    OTL (Turn a bit ON)

Reset								    OTU (Turn a bit Off)

END			END			END

A	C

D

B